P.O.BOX 100                                PHONE: (209) 537-4747

CERES, CA 95307               TOLL FREE: (800) 692-5844

www.west-mark.com                        FAX: (209) 537-1753

GENERAL

The following equipment shall be a new dust control tank truck.

PRODUCT: Non-potable water.

PRODUCT WEIGHT: 8.35 lbs./gallon.

VESSEL CODE: Non-Spec.

TANK OPERATING PRESSURE: Atmosphere.

TANK OPERATING TEMPERATURE: 125° F maximum

INTENDED AREA OF OPERATION: Specify _______________________________.

INTENDED USE: Highway and Unimproved roads.

CHASSIS

New Freightliner model M2 106 conventional cab, 2-door chassis:

BRAKES:	      Air type

DRIVE:		      4 X 2

GVWR:		      33,000 lbs

Front Axle:	           12,000 lbs Capacity

Rear Axle:	           21,000 lbs Capacity, Spring Suspension

Engine:	 	      Cummins ISB 250 HP and 660 lb/ft torque

Transmission:	      Allison 3500 RDS automatic

• Cab color to be White

• Battery disconnect switch

• Maintenance free batteries

• Remote mounted positive & negative posts for jumpstart

• Vertical tailpipe mounted at RH side

• Coolant rated to -34F

• Engine block heater

• Quick disconnect fitting with tire inflation kit.

• Front tow hooks, eye(s), or pin (application varies based GVWR)

• LH and RH grab handles

• Air horn

• Electric horn(s)

• Daytime running lights

• LH and RH convex mirrors mounted under primary mirrors

• Heater, defroster, and air conditioner

• One (1) each 12V power supply in dash

• Speedometer/Odometer capable of displaying in both miles and kilometer, with trip and hour meter.

• Adjustable tilt and telescoping steering column

• Back-up alarm

• AM/FM/WB radio with Bluetooth and Auxiliary inputs

• Cab suspension - Air Ride

• Air driver and fixed passenger seats

• Heated mirrors (flat and convex)

• Heated fuel/water separator

• Mud and snow tires on rear axle(s)

• Steel wheels

• Towing extended/roadside service warranty, 1 year/unlimited miles/km, $550 cap (U.S. Domestic only)

BODY

• 2,000 Gallon Non-Potable water painted steel tank.

• Heads, 3/16" A36

• Baffle(s) 3/16" A36.

• Baffle(s), Longitudinal 3/16" A36

• U-shaped subframe.

• Doubler plate, 3/16" A36

• Rubber pads between tank and frame.

• Tank Mounts, four (4).

• Manway, 24" circular

• Vortex Breaker, plate w/ screen

• Hydrant Fill, 2.5" w/ swivel swing spout

• Sight Gauge(s), two (2); one at rear and one at front w/ shut-offs.

• Ladder, one (1) rear mounted w/ handrails.

• Fenders, two (2) bonnet style tandem axle.

• Walkway, non-skid top of tank.

• Light Kit, D.O.T.  L.E.D.

• Conspicuity tape, D.O.T.

• Mud flaps, rear mounted.

• Paint, tank to be WHITE to match cab.

• Pump, water 750 gpm rated.

• PTO, air shift

• Driveline, 1000 series PTO shaft

• Controls, PTO Cab mounted.

• Pump plumbing, carbon steel.

• All plumbing components welded with groove couplers.

• Distribution manifold.

• Air valves for spray heads cab controlled

• Front Sprayers, two (2) with adjustable fan heads.

• Rear Sprayers, two (2) with adjustable fan heads.

• Side spray left hand side, duck bill with manual up/down operation

• Discharge, 1.5" ball valve driver's side.

• Control Switches, cab mounted.

• Rear in-frame storage area with lid.

• Bumper, rear per D.O.T.

• Paint, frame and plumbing components painted BLACK.

• Low point drains installed in all plumbing.

Warranty, unit shall be free from defects in material and workmanship for a period of ONE (1) year.

FOB: Phoenix, AZ