One (1)

Z1-05-1000

QW-ONLY

P.O.BOX 100                                PHONE: (209) 537-4747

CERES, CA 95307               TOLL FREE: (800) 692-5844

www.west-mark.com                        FAX: (209) 537-1753

S	One (1)

Z1-11-2001

QW-ONLY

GENERAL

The following equipment shall be a new West-Mark 3A sanitary tandem axle semi trailer.

One (1)

Z1-20-0436

QW-ONLY

OVERALL LENGTH: Approximately forty-three feet six inches (43' 6").

One (1)

Z1-30-0210

QW-ONLY

PRODUCT: Milk.

One (1)

Z1-32-0086

QW-ONLY

PRODUCT WEIGHT: 8.6 lbs./gallon.

One (1)

Z1-42-0000

QW-ONLY

OPERATING PRESSURE: Atmosphere.

One (1)

Z1-43-1650

QW-ONLY

OPERATING TEMPERATURE: One hundred sixty-five degrees Fahrenheit (165° F) maximum.

One (1)

Z1-50-0015

QW-ONLY

INTENDED AREA OF OPERATION: State of California.

One (1)

Z1-55-0010

QW-ONLY

INTENDED USE: Highway only.

One (1)

Z0-00-0200

QW-ONLY

TRACTOR

One (1)

Z2-05-0002

QW-ONLY

TRACTOR:

Customer supplied.

(Note: weights are based on tractor full of fuel and with driver.)

A=____        D=____        G=____

B=____        E=____

C=____        F=____

(Customer to supply chassis info. at time of order.)

If information is not supplied at time of order, West-Mark will use its standard typical setup.  Any information supplied after engineering package has been released to production, that requires set-up changes, will be done at additional charge to customer.

One (1)

Z0-00-0300

QW-ONLY

TANK / MANHOLES / VENTING

One (1)

Z3-02-677B

SA671A07

BARREL:

6700 Gallons round, straight barrel, with slope to rear. The barrel shall be made from 12 gauge T-304 stainless steel, #4 polish material. Polished finish to the inside.

One (1)

Z3-12-9991

QW-ONLY

Capacity does not include head outage.

Two (2)

Z3-20-0062

H3-7710341

HEADS: Two (2) each stainless steel, T-304 #4, 12 gauge, West-Mark air-dished and flanged.

WEST-MARK AIR DISHED 2B HEAD SPIN POLISHED TO #4 I.D.

Two (2)

Z9-07-0050

QW ONLY

_

IN HOUSE FORMING

Eight (8)

Z3-42-1113

H7-7310071

RINGS: Stainless steel channel sections, fully welded to shell.

ISSUED BY FEET 160' FOR MILK TANK

One (1)

Z3-44-SH10

QWZ344SH10

WINGS AND BOLSTERS: Shall be made of stainless steel.

One (1)

Z3-43-SA10

QWZ343SA10

SIDE FILLERS: Stainless steel with coupler plate adjustment rails.

One (1)

Z7-32-0120

QWZ7320120

_

GUSSETS, S/S AT REAR SUBFRAME BOLSTERS (TA ST TRAILER)

One (1)

Z3-46-0161

QW-ONLY

WELD FINISH:

INTERIOR: Weld finish to be W-5, 3-A Sanitary inside.

One (1)

Z3-46-2101

QW-ONLY

EXTERIOR: Weld finish to be W-0 outside.

One (1)

Z3-51-0130

MHXX1A01

MANHOLE:

One (1) each 18" Stainless steel, West-Mark, with removable, cleanable gasket around collar.

S	One (1)

Z3-51-0173

DECKPLATE:

Stainless steel, milk type around manhole with no standing area..

One (1)

Z3-60-0100

VT571110

VENTING:

One (1) Each spring loaded 3" Olsen vent(s) with ACME thread nylon nuts mounted in lid of manhole.

One (1)

Z3-60-0155

QW-ONLY

ANTI-SURGE CHECK VALVE FOR OLSEN VENT:

None. (Available as option upon request)

One (1)

Z3-60-0200

V2-6910100

VENTING:

One (1) Each 3" Runo vent(s) with ACME thread plastic nut mounted in lid of manhole.

One (1)

Z3-72-C707

QWZ372C707

INSULATION:

Barrel is insulated with 3" Polystyrene foam sheets, fiberglass at heads, ring tape at all rings.

Standard 1lb Density

One (1)

Z3-65-S707

QWZ365S707

JACKETING:

Belly wrapped 24 gauge bright finish stainless steel with lapped seams.

Two (2)

Z3-71-2010

H3-3710100

Jacket heads to be 20 Gauge stainless steel, lock seamed to jacket.

One (1)

Z8-04-2012

CPMLK20202

C.I.P.:

West-Mark 2-Ball stainless steel, bottom mounted with inlet at top of tank, inside manhole collar.

One (1)

Z8-04-8010

QW-ONLY

MANIFOLD: None. (Available as option upon request)

One (1)

Z8-04-8812

CPOPT2

INLET FITTING: To be 2" male acme thread.

One (1)

Z9-95-2005

IN HOUSE LABOR

_

HYDRO TEST (NON-CODE)

One (1)

Z0-00-0400

QW-ONLY

LADDERS / WALKWAYS / SPILLDAMS

One (1)

Z4-32-0210

L1-0111120

LADDERS:

Aluminum tubular with West-Mark heavy duty slip resistant steps.  Two (2); one located each side of deckplate. Step will be between 18” and 22” from the ground.  No grab rails (available as option upon request).

One (1)

Z0-00-0500

QW-ONLY

LIGHTS AND WIRING

One (1)

Z5-00-4950

ELECTRICAL SYSTEM:

System shall be 12-volt, 7-way, and vapor proof.  All lamps to be L.E.D. type and locations shall meet all D.O.T. requirements.

STOP/ TAIL/ TURN: Four (4) each model 44 type in 2-hole, stainless steel boxes; 2 red each side.

CENTER ID: One (1) each Truck-Lite model 15 stainless steel I.D. shall be mounted at rear per D.O.T.

MID TURN/ MARKER: One (1) each amber model 60 oval turn/ marker light shall be mounted @ center of unit (approx.) each side if required by D.O.T.

FRONT MARKERS: One (1) each amber model 30 light shall be mounted to the front corner of the trailer at 45° on each side of unit.

REAR MARKERS: One (1) each red model 30 marker lamp located at end of each light box.

One (1)

Z5-16-0998

QW-ONLY

SECURITY RINGS / MOUNT COVERS:  None. (Available as option upon request)

One (1)

Z5-20-1000

FLOOR STOCK

REFLECTORS:

To be mounted per D.O.T. specifications.

One (1)

Z5-25-1000

FLOOR STOCK

CONSPICUITY STRIPING:

To be installed at sides and rear per D.O.T. specifications.

One (1)

Z5-30-1200

LWXXXR03

FRONT RECEPTICAL:

Truck-Lite 7-way nosebox (model #50806) with 15 amp circuit breakers and front access to cable hookup, mounted on a stainless steel bracket between glad hands.

One (1)

Z0-00-0600

QW-ONLY

PLUMBING

One (1)

Z6-06-1340

OTXX1M15

VALVING:

One (1) each McFinn #60TTF 3" sanitary milk valve at rear with stainless steel dust cover and West-Mark stainless steel nut assembly.  Safety lock device on stem.  90° Outlet to roadside.

One (1)

Z8-12-0005

QW-ONLY

PUMP PACKAGE:

None. (Available as option upon request)

One (1)

Z0-00-0700

QW-ONLY

UNDERCARRIAGE

One (1)

Z6-08-0100

S2-0500560

APRON:

Stainless steel splash pan to be installed under valve at rear of trailer.

S	One (1)

Z7-10-0100

CAXX1A09

KICK PLATE:

To be installed at front of unit and to be of stainless steel construction.

KING PIN PLATE:

Adjustable upper coupler, mild steel.

48" Ride height, unless noted otherwise in tractor spec's above.

King Pin to be located: _____26__________

For tanks that require longer kick plate, use release # CAXX1A11.

One (1)

Z7-10-0900

QW-ONLY

WEAR PAD:

None.  (Available as option upon request)

S	One (1)

Z7-25-1125

S2-4520660

LANDING GEAR FRAME:

Frame to be stainless steel construction.

One (1)

Z7-26-0300

LGXXXB01

LANDING LEG SUPPORTS:

Pipe style from legs up to crossmember and to rear of subframe.

S	One (1)

Z7-26-0315

LGXXXEB1

Extra pipe supports added going forward up to frame and extra bracing between legs.

One (1)

Z7-27-0130

L2-3700940

LANDING GEAR:

JOST Aluminum square leg, 2-speed, steel sand shoes, 55,000 lb. lift capacity.

Crank located on curbside.

One (1)

Z7-30-1220

S2-6921450

REAR SUBFRAME:

Stainless steel construction for a tandem axle suspension.

S	Two (2)

Z7-35-1335

S3-3002920

SUSPENSION:

Two (2) RIDEWELL RAR 266 23K air ride integration system

_17_" ride height, weld-on wing style hangers, front shock absorbers

AXLES:

Two (2) 23K, 5-3/4" round 71-1/2" track

10 stud on 11.25" centers ADI hubs with HUB piloted mounting

HP spindles (same inner and outer bearings)

Synthetic semi fluid grease

Tire inflation prepped

BRAKES:

16-1/2" x 7", 28 spline S-Cam, balanced fused drums

16.5" x 7" HXS Abex 3030-197 non-asbestos brake lining

Ridewell's standard chambers and automatic slacks

Tone rings for ABS system

Four (4)

Z7-45-8000

QW-ONLY

DUST SHIELDS:

None. (Available as option upon request)

Four (4)

Z7-69-2351

W1-0110710

WHEELS:

OUTER: Four (4) Each Alcoa 22.5 x 8.25 Aluminum machine finish, 10-hole, 11-1/4" bolt circle, hub piloted.

Four (4)

Z7-69-2350

W1-0110710

INNER: Four (4) Each Alcoa 22.5 x 8.25 Aluminum machine finish, 10-hole, 11-1/4" bolt circle, hub piloted.

Eight (8)

Z7-70-2740

T2-0500220

TIRES:

Eight (8) Each 295/75R 22.5 CONTINENTAL HT3.

Eight (8)

Z7-70-9910

P3-5300310

_

(TIRE MOUNTING FEE)

(Qty to be = to the number of tires on unit)

Two (2)

Z7-71-0100

QW-ONLY

TIRE MANAGEMENT SYSTEM:

None. (Available as option upon request)

One (1)

Z7-75-1100

RBXX1A00

BUMPER:

Stainless steel non-code, 3" x 2" x 66" long.

(If underidement bumper is required, this item may be eliminated.)

S	One (1)

Z7-50-2120

BKX36543

BRAKE SYSTEM:

To be built to D.O.T. specifications, WABCO 4S/2M ABS system and Aluminum air tank(s).  ABS malfunction/ operation light located on front fender bracket of roadside rear fender, facing forward, per D.O.T.

S	One (1)

Z7-55-1950

QW ONLY

HEIGHT CONTROL (SUSPENSION SPECIFIC):

One (1) Each height control valve.

One (1)

Z7-55-3030

QW ONLY

DUMP VALVE:

Manual dump valve to be installed.

One (1)

Z7-85-2100

FDXX2A01

FRONT FENDERS:

Aluminum contour, ribs down.

One (1)

Z7-81-2100

FBX11A11

Stainless steel fender pipe mounting with U-bolts and under straps.

One (1)

Z7-90-2000

FKXX2A1124

Mudflaps at rear of front fenders to be WHITE, poly, anti-spray type.

One (1)

Z7-85-2150

FDXX2A11

REAR FENDERS:

Aluminum contour, ribs down.

One (1)

Z7-81-2200

FBX11A04

Stainless steel fender pipe mounting with U-bolts and under straps.

One (1)

Z7-90-2010

FKXX2A11

Mudflaps at rear of unit to be WHITE, poly, anti-spray type. Mudflap mounting to meet SAEJ682 standard.

For units with extended rear frame and W-M fenders, use 36" flaps (release # FKXX5A02).

One (1)

Z0-00-0800

QW-ONLY

MISC. ITEMS

One (1)

Z8-19-0020

FLOOR STOCK

TRANSPORT SECURITY SYSTEM:

Lugs for padlock/seal tab security system at Manhole, Outlet, C.I.P., Sample Box, and Pumpbox as applicable. Drilled lug nut handles for seal tab cable.

One (1)

Z9-95-0280

P3-5300520

CERTIFICATION:

California Department of Food and Agriculture inspection and sticker.

One (1)

Z8-44-0001

QW-ONLY

PLACARDS:

None. (Available as option upon request)

One (1)

Z8-96-5100

L3-0602220

CERTIFICATE HOLDER:

Betts PS-1 (#820012) document holder with spring.

Mounted on landing leg subframe (roadside).

One (1)

Z9-05-0002

QW-ONLY

PAINT:

None. (Available as option upon request)

One (1)

Z9-95-0101

F5-7700050

MANUALS:

One (1) each operation and maintenance manual(s) on CD-ROM shall be supplied and shipped with unit; complete with the following:

• Operator & maintenance manual

• Calibration chart (if applicable)

• Equipment parts list

• Misc. vendor literature / manuals

One (1)

Z9-95-0200

CALC9999

CALIBRATION:

Theoretical, in 1/4" increments, complete with charts.

(Available in Gallons, Liters, and Barrels)

One (1)

Z9-98-9992

QW-ONLY

WARRANTY:

Unit shall be guaranteed to be free from defects in material and workmanship, while under normal use and service by the original purchaser, for a period of ONE (1) year from the date unit is delivered.

One (1)

Z9-98-A030

Z9-98-9999

F.O.B.:

West-Mark

581 Industry Way

Atwater, CA 95301.

(F.O.B. destination available as option upon request.)

One (1)

Z9-98-999R

INSPECTION AND ACCEPTANCE:

West-Mark (CAGE: 880W5)

581 Industry Way

Atwater, CA 95301